Background

All four concrete mix design scenarios are Portland cement concrete (PCC) mixes, developed according to the NDOT Table 1002.02 concrete class requirements. Our group has selected classes 47B, 47BD, 47B-HE, and SF. These represent pavement, bridge deck, high-early-strength, and high-performance structural concrete applications, respectively.

Source

Standard specifications for Highway Construction – 2017 edition – NDOT

Link: https://dot.nebraska.gov/media/g4qp4y0d/2017-specbook.pdf

Scenario 1

- Pavement concrete

| Class of Concrete | Cementitious Materials Min. (lb/cy) | Air Content % Range (min-max) | Water-Cement Ratio Max | Required Strength Min. psi | Total Aggregate Min (lb/cy) | Total Aggregate Max (lb/cy) |
| --- | --- | --- | --- | --- | --- | --- |
| 47B | 564 | 6.5-9.0 | 0.45 | 3500 | 2850 | 3150 |

*lb/cy stands for pounds per cubic yard

Scenario 2

- Bridge deck concrete

| Class of Concrete | Cementitious Materials Min. (lb/cy) | Air Content % Range (min-max) | Water-Cement Ratio Mix | Required Strength Min. psi | Total Aggregate Min (lb/cy) | Total Aggregate Max (lb/cy) |
| --- | --- | --- | --- | --- | --- | --- |
| 47BD | 658 | 6.0-8.5 | 0.42 | 4000 | 2500 | 3000 |

Scenario 3

- High-early-strength concrete

| Class of Concrete | Cementitious Materials Min. (lb/cy) | Air Content % Range (min-max) | Water-Cement Ratio Max | Required Strength Min. psi | Total Aggregate Min (lb/cy) | Total Aggregate Max (lb/cy) |
| --- | --- | --- | --- | --- | --- | --- |
| 47B-HE | 752 | 6.0-8.5 | 0.40 | 3500 | 2500 | 3000 |

Scenario 4

- High-performance structural concrete

| Class of Concrete | Cementitious Materials Min. (lb/cy) | Air Content % Range (min-max) | Water-Cement Ratio Max | Required Strength Min. psi | Total Aggregate Min (lb/cy) | Total Aggregate Max (lb/cy) |
| --- | --- | --- | --- | --- | --- | --- |
| SF | 589 | 6.0-8.5 | 0.36 | 4000 | 2850 | 3200 |